LENTZ VALVE FOUR CYLINDER COMPOUND SPECIAL INSTRUCTIONS

This engine is a marine double compound expansion type using four cylinders, two high pressure and two low pressure. In fact, the engine consists of 2 steam engines mounted head-on to each other. The engine was used on tug boats in the Netherlands. The high pressure cylinders are operated 180 degrees from the low pressure cylinders. The front engine is 90 degrees from the rear engine. The advantage of this construction is that (reverse rod on max) the engine will always start on one of the 2 HP cylinders in every position. No auxiliary steam will be necessary. The high pressure exhaust is passed directly to the low pressure steam.(no receiver) The valve gear is two Bremme type, one driving the rear engine and the other the front engine. Long vertical cam driving rods are used to drive the high pressure cylinder steam cams from the Bremme gear. Horizontal cam rods connected the high pressure steam cams to the high pressure exhaust cams / low pressure steam cams. Cross shafts connect the low pressure steam cams to the low pressure exhaust cams. These cams operate the top and bottom Lentz type double ported poppet valves.

Two of these engines are still in use and in good shape on the two tugboats owned by the 'Haven Museum' in Rotterdam. With a piston stroke of one meter each tugboat produces 600 hp. Both tugboats have been modified from coal to oil burners. The last one in 2000.

Fuel consumption: small burners   150 liters/hr (firing up and slow ahead)

big burners   300 liters/hr  (full power)

The tugboats were to be sent to Russia. During World War Two everything changed. After the war they were needed in the Netherlands, so they were kept in the Netherlands.

Jan Haring

The Lentz valve is not shown, just a simple poppet valve.

This engine being a marine engine would have its exhaust discharged into a condenser so there would be no exhaust sound.

The engine has motion in three dimensional space. The program uses two dimensions of a plane. In order to place the program of the engine in two dimensions, the cranks, main rods, eccentrics, eccentric rods and the lower end of the long vertical cam rods have been rotated in space 90 degrees to place them in the two dimensional plane. Also in order to maintain approximately 90 degrees between the rear engine top end of the long cam rod and the cam driving lever, the cam driving lever angle is below the horizontal of the cam, by the amount of the Eccentric Offset Angle. Due to the long length of the vertical cam rods, the errors created when changing from three dimensions to two dimensions are too small to consider.